|  | DEPARTMENT OF COMPUTER SCIENCE & ENGINEERING THE NATIONAL INSTITUTE OF ENGINEERING (An Autonomous Institution) |  |
| --- | --- | --- |

TrailGuard

Batch No : B-12	Date : 13-08-2026

| USN | Name | Email-ID | Mobile Number |
| --- | --- | --- | --- |
| 4NI23CS076 | K Prakruthi Suresh | 2023cs_kprakruthisuresh_b@nie.ac.in | 8019234394 |
| 4NI23CS083 | Kishan R | 2023cs_kishanr_b@nie.ac.in | 9113943502 |
| 4NI23CS093 | M A Akshobhya Kashyap | 2023cs_maakshobhyakashyap_b@nie.ac.in | 7483063803 |
| 4NI23CS151 | Prasanna Basavaraj Indi | 2023cs_prasannabasavarajindi_b@nie.ac.in | 7483999983 |

Project Title: TrailGuard – Off-Grid Communication & Safety Network for Hiking Trails

Introduction: TrailGuard is an off-grid communication and safety platform built for hiking trails that fall outside cellular coverage. A small number of solar-powered LoRa mesh nodes are mounted at trail markers along the route; each node also functions as a Bluetooth Low Energy (BLE) peripheral, allowing a hiker's smartphone — running a lightweight companion app installed before the hike — to connect directly to nearby nodes without requiring Wi-Fi, cellular signal, or any additional hardware. Once connected, the hiker can view an offline trail map, log a routine check-in, or send a digitally signed SOS that is relayed node-to-node toward a ranger or trailhead station, even when no direct radio path exists at the moment the alert is sent. The project is scoped as a focused, low-cost pilot: a handful of nodes placed along one trail, validated with real hikers and real field conditions before any larger investment is considered.

Objectives: - Design and deploy a solar-powered LoRa mesh network of trail-marker nodes capable of multi-hop message delivery without relying on cellular or satellite infrastructure. - Develop a lightweight companion mobile app that connects to trail-marker nodes over Bluetooth Low Energy (BLE) to provide an offline trail map, check-in, and SOS functions. - Compute live bearing and distance guidance back to the trail by combining each node's pre-surveyed GPS coordinate with the hiker's phone GPS fix. - Implement store-and-forward message routing so that alerts are not lost when the mesh is temporarily broken. - Secure SOS and check-in messages against spoofing and replay using per-node digital signatures, verified on-device by the app. - Build a trailhead dashboard giving staff live visibility into SOS alerts, check-in logs, and node battery/solar health. - Validate real-world SOS latency, bearing accuracy, and battery life through field deployment on a single pilot trail before scaling to a wider area.

Software Requirements – - Meshtastic open-source firmware (adapted) for LoRa mesh routing and store-and-forward messaging. - Arduino / ESP-IDF for ESP32 node firmware development. - On-node BLE GATT service exposing node coordinate, check-in, and SOS characteristics to the companion app. - Native/cross-platform mobile app (e.g., Kotlin for Android, or Flutter) for the offline map, check-in, SOS interface, and bearing computation. - Python / Node.js for the trailhead dashboard backend. - SQLite for local logging of check-ins, node telemetry, and dashboard records. - A cryptographic signing library (e.g., Ed25519/ECDSA) for message authentication, implemented on-node and verified in-app.

Hardware Requirements – - ESP32 microcontroller with integrated Bluetooth Low Energy (BLE). - LoRa radio module (e.g., SX1276/SX1262). - Solar panel with charge controller and rechargeable Li-ion/LiFePO4 battery. - Weatherproof enclosure for trail-marker mounting. - Trailhead gateway device (Raspberry Pi or equivalent) with a LoRa interface. - Hiker's own smartphone with the companion app installed (no additional hardware required on the hiker's side).

Abstract: Hiking trails frequently pass through terrain with no cellular signal, leaving an injured or lost hiker with no way to raise an alert until they reach coverage or are reported missing, often hours later. Existing alternatives — satellite messengers, dedicated mesh-radio devices, and manual trailhead sign-in sheets — either shift cost and effort onto the individual hiker or only surface problems after the fact. TrailGuard addresses this gap by placing the intelligence in the trail itself: a small number of solar-powered nodes, each combining a long-range LoRa radio with a Bluetooth Low Energy interface, are mounted at trail markers to form a mesh network. A hiker installs a lightweight companion app before setting out; the app connects to nearby nodes over BLE and provides an offline trail map, a check-in function, and a digitally signed SOS, together with live bearing-and-distance guidance back to the trail computed from each node's known coordinate and the phone's own GPS fix. Store-and-forward routing ensures alerts are delivered even through temporary coverage gaps, while adaptive sleep scheduling lets nodes run for long stretches on solar power without compromising emergency response time. Built on an adapted open-source Meshtastic platform, the initial deployment targets 3–5 nodes on a single trail to validate multi-hop delivery, SOS latency, bearing accuracy, and battery performance under real field conditions before wider rollout across a forest or protected area.

Literature Survey (Minimum 5 recent research journals indexed in Scopus/Web of Science): Sl. No.; Author(s); Year; Title; Methodology; Limitation; Conclusion. 1; Rekha K R, Chinmayee Narayan, Harshitha Keshav, Pratheeksha H S, Deepika S N; 2025; Solar-Powered LoRa Mesh Network for Emergency Communication and Tracking During Disasters (IJARCCE, DOI: 10.17148/IJARCCE.2025.141266); Decentralized, ESP32-based solar-powered LoRa mesh with an offline web server for messaging and GPS tracking.; Not specified in the reviewed literature summary.; Validates self-forming mesh routing on solar power; directly informs TrailGuard's node hardware and offline-access design.. 2; Vaishnavi S, Ragavi R, Swathi V, Swegha C, Renuga M; Not specified; LoRa-Enabled Disaster Management and Emergency Communication (IJET, Vol. 12, Issue 2); Emergency-trigger and GPS-based LoRa alert system built on STM32 and ESP8266, operating without GSM or internet.; Not specified in the reviewed literature summary.; Provides a prototype-level SOS pipeline that parallels TrailGuard's alert relay.. 3; K. C. V. G. Macaraeg, C. A. G. Hilario, C. D. C. Ambatali; 2020; LoRa-based Mesh Network for Off-grid Emergency Communications (IEEE Conference Publication); Decentralized off-grid LoRa mesh using AODV routing that lets Bluetooth-only smartphones join the network without native LoRa hardware.; Provides no map-relative positioning or guidance for the user.; Establishes BLE-based phone access to a LoRa mesh; TrailGuard's contribution lies in bearing computation layered on a similar access model.. 4; Ana Rita Ortigoso, Gabriel Vieira, Daniel Fuentes, Luís Frazão, Nuno Costa, António Pereira; 2025; HaLert: A Resilient Smart City Architecture for Post-Disaster Based on Wi-Fi HaLow Mesh and SDN (arXiv preprint); Dual-mesh emergency architecture combining Wi-Fi HaLow with an SDN-controlled LoRa backbone.; Focused on urban smart-city post-disaster scenarios rather than off-grid hiking trails.; Achieves message success rates near 95% in real urban trials, offering a reliability benchmark for TrailGuard's node-to-node relay.. 5; Y. Liu et al.; 2025; Balancing Quality of Service and Lifetime in Solar-Powered IoT via Hyper-Adaptive Duty Cycling (IJRER); Adaptive duty-cycling scheme that adjusts sleep/wake behaviour to harvested solar energy, data backlog, and battery state.; Not specified in the reviewed literature summary.; Directly applicable to balancing node uptime against battery life without slowing emergency response.. 6; Muladi, Hisyam Wijaya, Singgih Dwi Prasetyo, Shipun Anuar Hamzah, Abd Kadir Mahamad; 2024; LoRa Mesh-Based IoT GPS Tracking System for Mountain Climbers (IJSSE, Vol. 14, No. 6, DOI: 10.18280/ijsse.140610); LoRa-mesh GPS tracker for climbers that auto-transmits coordinates every 5 minutes.; Tracking-only baseline; does not implement fixed-node bearing guidance or signed SOS/check-in.; Identifies SOS alerting as a natural future extension; TrailGuard builds past this tracking-only baseline.. 7; M. Chung, Y. Lu, C. Lee, S. Kuo, L. Chen; 2024; A Wearable IoT-Based Rescue System Using LoRa Mesh Network and Physiological Monitoring for Mountain Emergency Response (IEEE GCCE Proceedings); Pairs LoRa mesh with wearable biometric sensing for climber rescue.; Relies on a device the hiker must wear and carry, unlike TrailGuard's fixed-infrastructure, phone-only access model.; Confirms the mesh-for-mountain-safety design space for LoRa-based rescue systems.. Research Gap: Across the surveyed work, LoRa-mesh safety systems either track a hiker's own live position for third parties to view (Rekha et al.; Muladi et al.) or pair the mesh with a wearable biometric device the user must carry (Chung et al.); Macaraeg et al. shows BLE-based phone access to a LoRa mesh is achievable but adds no positioning capability; and RSSI-only localization approaches used elsewhere for rescue typically carry tens-of-meters to hundreds-of-meters error. None combine fixed, pre-surveyed trail-marker coordinates with the hiker's own phone GPS to compute a live bearing and distance back to the trail. This bearing-guidance mechanism — not the mesh, solar, BLE access, or SOS-relay components individually — is the specific contribution TrailGuard adds to the existing literature, and is the intended focus of the paper to be written after project completion.

Existing System & its Drawback: Satellite messengers (e.g., Garmin inReach, SPOT) cost USD 300–500 plus a recurring monthly fee per person and give trail operators no shared visibility into who is on the trail. Dedicated mesh-radio devices are cheaper but still require every hiker to buy, charge, and remember to carry a separate device. Manual trailhead sign-in sheets and periodic ranger patrols only surface problems after the fact — often hours later, sometimes after dark or after weather has changed. None of these approaches make use of the smartphone a hiker is already carrying, and none give staff real-time visibility into hiker check-ins or network health.

Proposed System & Advantages: - Places the intelligence in the trail itself via solar-powered LoRa mesh nodes, so hikers need no extra hardware — only their own smartphone with the companion app installed before the hike. - Off-trail bearing guidance: each fixed node broadcasts its own pre-surveyed GPS coordinate over BLE; the app combines this with the phone's own GPS fix to compute a live bearing and distance back to the trail — an approach not found in the surveyed literature, which relies on tracking the hiker's own position or on RSSI-based proximity rather than two independent, known coordinates. - A lightweight companion app, installed once before the hike, connects to any node along the route via Bluetooth Low Energy — no Wi-Fi or cellular signal required at any point. - Store-and-forward mesh routing ensures SOS and check-in messages are delivered even through temporary radio gaps. - Digitally signed messages protect against spoofed or replayed alerts, keeping the system trustworthy for emergency response. - Adaptive sleep scheduling — sunny nodes stay awake as the mesh backbone, shaded nodes deep-sleep but wake instantly on SOS — extends battery life without slowing emergency response. - A trailhead dashboard gives staff live visibility into SOS alerts, check-in activity, and node battery/solar health, which sign-in sheets and periodic patrols cannot offer. - Built on an adapted, open-source Meshtastic platform rather than a custom network stack, shortening build time and directing effort toward the app, bearing computation, security design, and real field validation.

Map the proposed system to the appropriate Sustainable Development Goal (SDG): SDG No. 3: Ensure healthy lives and promote well-being for all at all ages. TrailGuard directly supports SDG 3 by reducing emergency-response time for injured or lost hikers in areas without cellular coverage, through real-time SOS relay, bearing guidance back to the trail, and live check-in visibility for trailhead staff. Secondary alignment — SDG No. 9: Build resilient infrastructure, promote inclusive and sustainable industrialization, and foster innovation. The solar-powered, self-forming mesh network is low-cost, off-grid resilient infrastructure that can be extended to other remote or under-served areas.

Guide Details: Name : Janardhana Swamy G B Designation : Assistant Professor Mobile Number : 70199 84240

College Address : No 50, Koorgalli Village, Hootagalli Industrial Area, Next to BEML, Mysuru, Karnataka, 570018